計算機組織 第三次考試 (1/12 10:10~12:00)

- (7%) The Average Memory Access Time equation (AMAT) has three components: hit time (HT), miss rate (MR), and　miss penalty (MP). For each of the following cache optimizations, indicate which component of the AMAT equation is improved (give your answer in terms of HT, MR, or MP for each question below).

- Using a second-level cache

- Using a direct-mapped cache

- Using a 4-way set-associative cache

- Using a virtually-addressed cache

- Performing hardware pre-fetching using stream buffers

- Using a non-blocking cache

- Using larger blocks

- (3%) Which of the following techniques are aimed at reducing the cost of a miss for the cache system

(a) a larger block size

- the addition of a second level cache,

- the use of a TLB

- early restart with critical word first

- a writeback buffer

- increased associativity,

- (7%) Calculate the performance of a processor taking into account stalls due to data cache and instruction cache misses. The data cache (for loads and stores) has a 92% hit rate and 124-cycle miss penalty. The instruction cache has a 90% hit rate and 50-cycle miss penalty. Assume the base CPI (ideal cache) is 2, and 30% of instructions are loads and stores.

  - Calculate actual CPI of the processor.

- Double clock rate, the time to handle cache miss does not change. What is the new CPI and how much faster will the processor be with the same miss rates?

- (12%) Given a cache with a total data size of 64 words, 2-way set associative, two-word blocks and LRU replacement. For a series of references as word addresses below, answer the following questions.

26, 27, 28, 10, 11, 58, 59, 26, 91, 59 (decimal)

- (3%) Please label each reference as a cache hit or a miss. Also, label each miss as being either a compulsory miss, a capacity miss or a conflict miss.

- (3%) For each type of miss, give one technique that can be used to reduce the miss rate.

- (3%) Assuming that the cache design is changed to 4-way set associative with four-word blocks, label each reference as a hit or a miss.

- (3%) For each miss in (a) which became a hit in (c), please explain what technique used in the new configuration causes the change (i.e., miss became hit).

- (14%) Consider a memory system with the following parameters:

- Translation Lookaside Buffer has 512 PTEs (page table entries) and is 2-way set associative.

- 64Kbyte L1 Data Cache has 128-byte blocks and is also 2-way set associative.

- Virtual addresses are 64-bits and physical addresses are 28 bits.

- 8KB page size

- Status fields in data cache, TLB, and page table use 4 bits (1 valid bit, 1 dirty bit, 2 LRU bits), respectively.

Below are diagrams of the cache and TLB. How many bits are used for each of fields (A~H) respectively? How large is the data cache, TLB, and page table, respectively? Please give all your answers in bits.

| L1 Cache | TLB |
| --- | --- |

- (16%) In a 16-bit byte address system, given a two-dimension array A[7][3] with 4-byte(1-word) element size, and the memory address (in byte) of each element A[i][j] are listed below:

A[0][0]: 004816

A[0][1]: 004C16

A[0][2]: 005016

A[1][0]: 005416

...

For a direct-mapped cache with 4-word blocks and 16-word total data size, answer the following questions.

- (3%) List the tags for the blocks which contains the following array elements respectively: A[0][2], A[3][0], A[6][2].

- (5%) Assume the cache is empty initially, give the cache hit/miss sequence of first 10 steps when accessing all the elements of array A by row-major.

- (5%) Assume the cache is empty initially, give the cache hit/miss sequence of first 10 steps when accessing all the elements of array A by column-major.

- (3%) For questions (b) and (c), which one has the higher cache hit rate? please explain your answer.

- (18%) Assume 4KB pages, a four-entry fully associative TLB, and true LRU replacement. If pages must be brought in from disk, increment the next largest page number. Given a series of virtual addresses below, the initial state of the TLB, and the page table, answer following questions

virtual address: 35A0, 24EC, 78F4, 4AC0, 55A6, 94DE, 410D

  - Show the final state of the system (both TLB and page table)..

  - For each reference, list the translated physical address, and label it as a hit in the TLB (H), a miss in TLB but hit in page table (M), or a page fault (PF). (Note: all the addresses, tags, PPNs are shown in hexadecimal format)

|  |  |  | Page table | valid | PPN |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 5 |
| TLB |  |  |  | 0 | disk |
| valid | Tag | PPN |  | 0 | disk |
| 1 | B | C |  | 1 | 6 |
| 1 | 7 | 4 |  | 1 | 9 |
| 1 | 3 | 6 |  | 1 | C |
| 0 | 4 | 9 |  | 0 | disk |
|  |  |  |  | 1 | 4 |
|  |  |  |  | 0 | disk |

Express your answer of question (b) as the following table.

| Virtual address | 35A0 | 24EC | 78F4 | 4AC0 | 55A6 | 94DE | 410D |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H or M or PF |  |  |  |  |  |  |  |
| Physical address |  |  |  |  |  |  |  |

- (4%) DMA

- Why is DMA an improvement over CPU programmed I/O?

- When would DMA transfer be a poor choice?

- (9%) Suppose we have a disk which rotates at 3600RPM. The advertised average seek time is 12ms, the sector size is 512 bytes, the transfer rate is 35MB/sec, and the controller overhead is 5.5ms. Assume that the disk is idle so that there is no waiting time. Answer the questions below.

- What is the average time to read a single sector?

- What is the average time to read 8 KB in 16 consecutive sectors in the same cylinder?

- What is the average time to read 8KB in 16 random sectors on this disk?

- (10%) Consider the following two RAID configurations below:

| Configuration 1 | Configuration 2 |
| --- | --- |
| Type: RAID 0 Block size: 32KB Disk 1: 80GB capacity, 180MB/s for read, 120MB/s for write Disk 2: 80GB capacity, 180MB/s for read, 120MB/s for write | Type: RAID 1 Block size: 128KB Disk 1: 500GB capacity, 100MB/s for read, 80MB/s for write Disk 2: 500GB capacity, 100MB/s for read, 80MB/s for write Disk 3: 500GB capacity, 100MB/s for read, 80MB/s for write |

Assume the data are all block-aligned, please answer following questions (a)~(d) for the Configuration 1 and (e)~(j) for the Configuration 2.

- How much space (in GB) can be used?

- What is the speedup when reading a 64KB continuous data, compared with only one disk?

- What is the maximum writing rate(MB/s) in this configuration?

- If each disk has a probability P to break down in a year, what is the probability to miss the data in one year?

- How much space (in GB) can be used?

- What is the speedup when reading a 256KB continuous data, compared with only one disk?

- What is the speedup when reading a 512KB continuous data, compared with only one disk?

- What is the maximum writing rate(MB/s) in this configuration?

- At least how much disk should alive, so the data will not miss?

- If we change the type to RAID 5, answer the question (e) and (i).

請寫上姓名學號然後隨答案卷一起交回,未交回題目卷者視同缺考。

姓名：　　　　　　　　　學號：

祝大家新年快樂囉！

～　ＥＮＤ　～